Half-mask respirator FILTROSTAT FS CODE 823 V FFP2 NR D

Category: FS 800 Filtrostat

Class: P2

PEL: 10 x OEL

Standard: EN 149:2001, meets the requirements of the long oil mist test

Certificate: issued by CIOP PIB (Central Institute for Labour Protection - National Research Institute) compliant with EU 2016/425.

She won the GOLD MEDAL of the SAWO 2003 Fair in Poznań.
The product was awarded the “NOW POLSKA” emblem in the 14th edition of the competition.

Application:

solid particles of medium toxicity, dusts of asbestos, copper, barium, titanium, vanadium, chromium, manganese, hard wood dusts, coal dust containing more than 10% of free silica, mining industry, chemical industry, metallurgy, shipyards, hard woodworking.

Additional advantages:

Compact, anatomical shape, soft and pleasant to the touch. The respirator is covered with a special layer of protective cloth, which extends its life. Thanks to the valve placement on the side of the half mask, it is possible to fold the respirator and pocket it without damage during work breaks.

Useful:

Respiratory tract protection against dust, solid particle aerosols, water-based aerosols and aerosols with dispersed liquid phase, up to 10 x OEL.

Composition:

Half-mask dome:
- Protective non-woven fabric
- Electret needled non-woven fabric
- Non-woven polypropylene melt blown fabric
- Protective non-woven fabric
Additional elements:
- Exhalation valve
- Nose clip for sealing the half mask in the nose area
- Polyethylene foam nose seal
- Head strap clips for length adjustment
- Head straps