FS-913V A FFP1 NR D half mask respirator

Category: 900 A SERIES – budget half mask respirators

Class: P1

OEL: 4 x OEL

Standard: EN 149 2001, meets the requirements of the long oil mist test

Certificate: issued by CIOP PIB (Central Institute for Labour Protection - National Research Institute) compliant with EU 2016/425

Thanks to production automation and streamlining, the 900 series of half mask respirators can offer attractive pricing. They provide a budget-friendly alternative to disposable respiratory protective equipment available on the market while maintaining high product quality.

Application:

Non-toxic dusts, agricultural industry, food industry, quarries, cement plants, particularly for the following dusts: calcium carbonate, natural and synthetic graphite, gypsum, chalk, cement, plaster, marble, zinc oxide, vegetable pollen, cellulose, sulphur, cotton, ferrous metal filings, coal dust with below 10% of free silica.

Additional advantages:

Compact, anatomical shape, soft and pleasant to the touch. The half mask respirator is covered with a special layer of fleece, which extends its lifespan. The exhalation valve facilitates the exhalation of air. Thanks to the valve placement on the side of the half mask, it is possible to fold the respirator and pocket it without damage during work breaks.

Useful:

For respiratory tract protection against harmful effects of air pollution in the form of solid particles and/or aerosol-forming liquid particles (dusts, vapours, mists) up to 4 x OEL.

Design:

Half-mask dome:

- Protective non-woven fabric
- Structural non-woven polypropylene fabric
- Non-woven polypropylene melt blown fabric
- Protective non-woven fabric
Additional elements:
- Exhalation valve
- Nose clip for sealing the half mask in the nose area
- Polyethylene foam nose seal
- Head straps