Disposable half mask FS-930 FFP3 NR

Category: SENSITEC 900 SERIES

Class: P3

NDS: 30 x OEL.

Standard: EN 149 2001, meets the requirements of the long oil mist test

Certificate: Issued by CIOP “Central Institute for Labour Protection” compliant with EU 2016/425

Thanks to production automation and streamlining, the 900 series half mask respirators can offer attractive pricing. They provide a budget-friendly alternative to disposable respiratory protective equipment available on the market while maintaining high product quality.

Application:

High concentrations of respirable dusts; use for welding and soldering; protection against dusts containing: beryllium, antimony, arsenic, cadmium, cobalt, nickel, radium, strychnine and radioactive particles.

Additional advantages:

Compact, anatomical shape, soft and pleasant to the touch. The half mask respirator is covered with a special layer of fleece, which extends its lifespan. Thanks to the shape of the half mask, it is possible to fold the respirator and pocket it without damage during work breaks.

Useful:

For respiratory tract protection against harmful effects of air pollution in the form of solid particles and/or aerosol-forming liquid particles (dusts, vapours, mists) up to 30 x OEL.

Composition:

Half-mask dome:
- Protective non-woven fabric
- Structural non-woven polypropylene fabric
- Non-woven polypropylene melt blown fabric
- Protective non-woven fabric
Additional elements:
- Nose clip for sealing the half mask in the nose area
- Polyethylene foam nose seal
- Head strap clips for length adjustment
- Head straps